North Scottsdale Primary Suite + Ensuite Spec

Design Intent • Contractor Scope • Bid Standard

Primary Bedroom, Closet, and Ensuite Bath Remodel | Base Scope: Inside Existing Envelope

# 1. Executive Summary

Core vision Create a warm Sonoran Resort Modern primary suite: a calmer, larger-feeling bedroom; a true spa-level ensuite with freestanding tub + large shower if feasible; a private water closet; and a closet strategy that preserves luxury function without relying on an exterior addition.

Base target: Option B - Resort Spa Upgrade Plan. Use the existing bath plus adjacent closet zone as the planning opportunity, relocate plumbing where it materially improves value, and replace any lost closet function with a high-quality wardrobe/closet solution.

Excluded from base scope: the prior 5-8 ft exterior primary bath extension toward the pool. Exterior expansion may be priced only as a future alternate if requested, not included in base bidding.

Important limitation: This document is a contractor bid/control spec and design-performance standard. It is not a stamped engineering plan or permit set. Final dimensions, plumbing routes, structural conditions, and code requirements must be field verified before construction.

# 2. Known Existing Conditions / Planning Inputs

| Item | Bid / Design Assumption |
| --- | --- |
| Primary bedroom | Approx. 14.5 ft x 14.5 ft existing footprint. Potential to reclaim approx. 3 ft of hallway length if ROI and layout support it. |
| Possible bedroom re-balance | If the bedroom wall is moved and bath/closet walls are rebalanced, a possible target could be around 17.5 ft x 12 ft. This must be tested by designer; longer is not automatically better if the room becomes narrow or awkward. |
| Current ensuite bath | Approx. 8.5 ft deep x 10 ft long. Existing layout is not luxury-grade. |
| Adjacent closet | Approx. 8.5 ft deep x 7 ft long. This can be partially or substantially absorbed into the bath if replacement closet function is solved. |
| Combined bath + closet planning zone | Approx. 8.5 ft deep x 17 ft long if combined. This is the key opportunity zone. |
| Skylight | No skylight in this suite. The spa effect must come from layout, lighting, tile, stone, glass, and material warmth. |
| Plumbing | Plumbing may be relocated if it materially improves the bath layout and perceived value. Contractor must price plumbing relocation clearly. |
| Exterior addition | Excluded from base scope. |

Planning rule Do not make the bath bigger by destroying primary-suite closet function. The winning layout creates a stronger bath and a believable luxury closet solution.

# 3. Design Style: Sonoran Resort Primary Suite

The suite should feel like a high-end North Scottsdale resort residence: warm, calm, architectural, and spa-like. The luxury should come from proportion, material restraint, lighting, tile execution, glass, millwork, and clean detailing - not decorative clutter.

| Element | Target Specification |
| --- | --- |
| Walls | Smooth warm plaster-like finish; warm off-white, bone, sand, or limestone tone. |
| Flooring | Bedroom: warm engineered wood preferred unless whole-house continuity requires tile. Bath: large-format limestone-look porcelain or honed natural stone-look tile. |
| Bath tile | Warm limestone, travertine, soft taupe, sand, ivory, or subtle stone-look large-format tile. Avoid busy patterns. |
| Cabinetry / closet | White oak, walnut, warm mushroom, taupe, or blackened oak. Soft-close drawers; finished interiors; no wire-rack rental look. |
| Counters | Quartz, quartzite, or honed warm stone. Subtle movement. No busy speckled granite by default. |
| Metal finishes | Bronze, blackened bronze, brushed nickel, polished nickel, or aged brass in restrained use. |
| Lighting | Warm 2700K-3000K, layered, dimmable, with vanity, shower, tub, and closet/task lighting handled intentionally. |

Avoid: gray flip finishes, farmhouse lighting, heavy Tuscan, shiny generic marble, built-in Jacuzzi as the focal point, cold white/gray bath, cheap chrome everywhere, busy granite, and exposed or afterthought closet systems.

# 4. Spatial Program / Required Final Suite Functions

| Function | Requirement |
| --- | --- |
| Bedroom | King-bed capable, calm, larger-feeling, with strong wall composition and no awkward leftover circulation. |
| Bath | Resort spa bath with freestanding tub + large shower if feasible, double vanity, private WC, warm stone/tile, and high-quality lighting. |
| Shower | Large walk-in shower; bench/ledge, niche, wall shower, handheld, optional rain head; frameless glass. |
| Tub | Freestanding sculptural tub if it fits naturally; not a forced tub and not a built-in Jacuzzi by default. |
| Toilet | Private enclosed water closet desired if feasible without wrecking layout. |
| Closet | Closet function must be preserved or rebuilt through remaining closet, wardrobe wall, built-ins, or hybrid system. |

# 5. Layout Options to Price / Study

Base direction Option B is the preferred design target and should be treated as the base design study. Option A is acceptable if Option B loses too much closet function or costs disproportionately more. Option C is a budget fallback only.

| Option | Description / Bid Logic |
| --- | --- |
| Option B - Resort Spa Upgrade Plan (Preferred Base Target) | Use a substantial portion of the adjacent closet footprint to create a true spa bath in the approximate 8.5 ft x 17 ft planning zone. Target: freestanding tub + large shower + double vanity + private WC. Replace lost closet function with wardrobe wall / hybrid built-in closet solution. Plumbing relocation allowed. Highest perceived-value option if closet is solved well. |
| Option A - Balanced Bath / Closet Plan (Acceptable) | Expand the bath into part of the closet while preserving more existing closet volume. Target: large shower, double vanity, private WC if feasible, tub only if it fits naturally. Less dramatic than Option B, but lower risk to closet function and budget. |
| Option C - Closet-Priority Plan (Fallback) | Preserve most closet area and upgrade bath more surgically. Likely shower-focused; tub may be omitted. Use only if materially cheaper or if structural/MEP constraints make A/B inefficient. |

# 6. Primary Bedroom + Hallway Reclaim Strategy

The bedroom is currently approximately 14.5 ft x 14.5 ft. Reclaiming hallway/circulation space may add roughly 3 ft of length and create a possible long rectangle around 17.5 ft x 12 ft if walls are rebalanced. This can be valuable, but only if it creates a better bedroom composition.

Reclaim hallway space only if it creates at least one of the following:

a stronger king-bed wall with balanced nightstands and lighting

a credible wardrobe wall or built-in closet zone

a calm entry sequence into the suite

a small sitting/bench moment that photographs well

a better relationship between bedroom, closet, and bath

Do not reclaim hallway space if it creates:

a narrow awkward bedroom corridor

bad bed clearances

a closet wall that looks like an afterthought

confusing circulation to the bath

cost with no visible ARV payoff

Bedroom proportion standard A 17.5 ft x 12 ft bedroom can work if planned intentionally. It is not automatically better than 14.5 ft x 14.5 ft. The designer must confirm furniture layout, bed wall, closet strategy, and circulation before approving wall moves.

# 7. Ensuite Bath Layout Requirements

The exact layout is not final, but any final plan should be judged against these dimensional and functional targets.

| Component | Target Standard |
| --- | --- |
| Large shower | Minimum target 42 in x 60 in. Preferred 48 in x 72 in or larger if layout allows. Include bench/ledge, niche, handheld, wall head, optional rain head, frameless glass. |
| Freestanding tub | Target 60-66 in long x 30-34 in wide. Include comfortable clearance around tub, floor or wall tub filler, and privacy if near window. Do not force tub if it destroys shower, WC, or closet function. |
| Double vanity | Minimum target 72 in. Preferred 84-96 in if space allows. Use drawers, storage, stone/quartz counter, sconces/vertical vanity lighting, and clean outlet placement. |
| Private WC | Minimum target 36 in wide x 60 in deep; preferred 42 in wide where possible. Include exhaust, proper door swing or pocket door, and a non-claustrophobic feel. |
| Circulation | 36 in minimum in primary bath circulation; 42 in preferred at key paths where feasible. |
| Closet function | Must be replaced or preserved through a credible wardrobe/closet solution. No wire-shelf rental closet as final luxury condition. |

# 8. Arrangement Concepts for Option B

The design team should test these concepts before finalizing. Contractor should price plumbing/framing flexibility rather than assuming all fixtures stay where they are.

| Concept | Description |
| --- | --- |
| B1 - Linear Spa Bath | Use the 8.5 ft x 17 ft combined zone as a long spa composition. Place vanity on one long wall, tub/shower sequence on the opposite or far wall, private WC at one end. Strongest if there is a clean entry and no pinched circulation. |
| B2 - Tub + Shower Feature Wall | Create a dramatic tub/shower wall at the end of the bath, with double vanity along the side and WC tucked/private. Best for listing photos if the tub and shower can be seen as a composed focal point. |
| B3 - Wet-Room Upgrade | Tub and shower share one waterproofed glass enclosure or wet zone. High-end look, but only use if waterproofing, slope, glass, ventilation, and space are excellent. Price as an alternate if uncertain. |
| B4 - Shower-First Luxury Bath | If tub creates too much compromise, make the shower exceptional and preserve more closet. This is acceptable only if the bath still feels luxury-grade and spa-like. |

Design bias The best ARV image is likely a large shower + freestanding tub + private WC + real closet solution. But a forced tub in a cramped bath is worse than an exceptional shower and better storage.

# 9. Closet / Wardrobe Strategy

Closet planning is not secondary. If the existing 8.5 ft x 7 ft closet is absorbed into the bath, the suite must receive a believable replacement closet strategy.

| Strategy | When to Use |
| --- | --- |
| Preferred hybrid strategy | Use part of existing closet footprint for bath expansion and replace lost storage with a wardrobe wall or built-ins in the bedroom/reclaimed hall zone. Balances spa bath and closet function. |
| Full wardrobe-wall strategy | Use most/all existing closet for bath and build a custom wardrobe wall in the bedroom. Best if bedroom wall length supports it and the millwork looks intentional. |
| Preserved closet strategy | Keep more existing closet and upgrade bath less aggressively. Use only if bath still reaches luxury standard or if cost savings are meaningful. |

Minimum closet quality standard:

finished built-in or high-quality closet system

hanging, drawers, shelves, and shoe/storage zones planned intentionally

soft-close drawers if built-in

integrated lighting if feasible

no exposed raw edges

no wire shelving unless specifically owner-approved as a downgrade

# 10. Material + Finish Schedule

| Element | Specification |
| --- | --- |
| Bedroom floor | Recommendation: warm engineered white oak or warm natural wood tone. Alternative: continue public limestone-look tile if whole-house continuity is preferred. Avoid gray LVP. |
| Bath floor | Large-format honed limestone-look porcelain or warm natural stone-look tile. Matte, slip-conscious finish. |
| Shower walls | Large-format porcelain, slab-look porcelain, limestone-look tile, or subtle stone. Keep grout minimal and layout intentional. |
| Shower floor | Mosaic or small-format tile compatible with slope, or linear-drain-compatible large format if installer can execute correctly. |
| Vanity | White oak, walnut, warm mushroom/taupe, or floating/furniture-style vanity. Soft-close drawers. |
| Countertops | Warm quartz, quartzite, honed limestone-look slab, or subtle stone. No busy speckled granite by default. |
| Walls | Smooth warm off-white/bone/sand plaster-like finish. Moisture-appropriate paint in bath. |
| Metals | Bronze, blackened bronze, brushed nickel, polished nickel, or aged brass used sparingly. Keep finish palette consistent. |
| Glass | Frameless shower glass, 3/8 in or 1/2 in where appropriate, minimal hardware. |
| Lighting | Layered warm lighting: vanity sconces, recessed general, shower-rated light, tub accent, closet lighting. 2700K-3000K. |

# 11. Contractor Base Scope of Work

Contractor shall price the following as the base primary suite scope:

Field verify all dimensions, wall locations, plumbing routes, electrical, HVAC, ceiling/roof conditions, window/exterior wall locations, and structural constraints.

Demolish existing primary bath/closet finishes and affected non-structural partitions as required for the approved interior reconfiguration.

Price Option B as the preferred base study: expanded spa bath using bath + closet planning zone, with replacement closet/wardrobe solution.

Relocate plumbing as required for the approved layout, including shower, tub, vanity, and toilet, where it materially improves the bath value.

Frame new walls, openings, blocking, niches, benches, shower curbs/curbless transitions, WC enclosure, and closet/wardrobe supports as required.

Install complete shower waterproofing system, pan, drain, tile substrate, tile, grout, sealants, and frameless glass.

Install freestanding tub if approved by final layout; otherwise price tub as alternate or omit if it compromises ROI.

Install double vanity, countertops, sinks, faucets, mirrors, lighting, and accessories.

Install private water closet if feasible, including toilet, exhaust, lighting, and door/pocket-door hardware.

Install closet/wardrobe system per selected strategy.

Modify electrical, plumbing, HVAC, exhaust, lighting, switches, outlets, and ventilation as required.

Install flooring, baseboards/trim, smooth drywall finish, paint, and final cleanup.

Base scope excludes: exterior bath addition, new Jacuzzi, major roof changes, appliance/furniture/staging, and any work not listed or approved in writing.

# 12. Workmanship / Acceptance Standards

## Shower + Waterproofing

Required:

Use one complete waterproofing system; do not mix systems unless manufacturer-approved.

Waterproof shower floor, walls, curb, bench, niche, corners, penetrations, and valve openings.

Slope shower pan, niche sill, bench, and curb top correctly toward drain/wet side.

Perform documented flood test before tile installation.

No mastic in wet shower areas; no tile over unapproved substrate.

No standing water after normal use.

Not acceptable:

No flood test; waterproofing only the pan; flat niche/bench; penetrated curb without approved detail; shower floor pooling; crooked niche.

## Tile

Required:

Approve layout before setting tile.

Avoid visible sliver cuts at focal walls, tub wall, shower niche, and vanity wall.

Use correct mortar/trowel/back-buttering for large-format tile; no spot-bonding.

Maintain consistent grout joints; remove grout haze; caulk changes of plane with matching flexible sealant.

Prep and flatten substrate before tile; do not use thinset to hide major floor waves.

Not acceptable:

Hollow tile, obvious lippage, crooked grout lines, random layout, sliver cuts in primary sightlines, grout haze.

## Vanity + Millwork + Closet

Required:

Cabinetry/closet built-ins level, plumb, square, and aligned.

Consistent reveals; soft-close hardware; finished exposed ends; clean scribe to walls.

Hardware aligned; drawers operate smoothly; no raw exposed edges.

Vanity plumbing must not block drawers unless approved in layout.

Not acceptable:

Misaligned doors/drawers, raw cabinet edges, crooked hardware, filler strips hiding poor layout, damaged finish.

## Plumbing + Fixtures

Required:

Fixture locations approved before rough-in.

Shower valves set at correct depth; tub filler aligned; drains centered/aligned where design requires.

Accessible shutoffs; no leaks; correct venting and drainage.

Not acceptable:

Off-center fixtures, vanity plumbing interfering with storage, slow drainage, leaks, crooked trim plates.

## Drywall + Paint

Required:

Smooth plaster-like finish in bedroom/bath affected areas.

No visible seams, patch lines, texture mismatch, fastener pops, or wavy corners under normal light.

Primer and two finish coats minimum where appropriate; clean cut lines.

Not acceptable:

Orange peel called smooth, visible patches, paint flashing, rough repairs around new walls/doors/vanity.

## Lighting + Electrical

Required:

Lighting layout approved before rough-in.

Warm 2700K-3000K lighting; dimmers where appropriate.

Vanity lighting symmetrical; shower light wet-rated; tub lighting safe/code-appropriate; closet lighting planned.

Outlets located for grooming, charging, and code compliance without disrupting tile/mirror composition.

Not acceptable:

Cold blue lighting, off-center fixtures, exposed cords, random switch locations, outlets chopped through focal tile.

# 13. Required Bid Breakout

| Bid Category | Required Breakout |
| --- | --- |
| General conditions / protection / haul-off | Yes |
| Primary suite demo | Yes |
| Framing / wall modifications | Yes |
| Plumbing relocation | Yes |
| Electrical / lighting | Yes |
| HVAC / exhaust ventilation | Yes |
| Shower waterproofing + pan + drain | Yes |
| Tile labor + materials allowance | Yes |
| Freestanding tub + filler | Separate line / allowance |
| Double vanity + countertop | Yes |
| Toilet / private WC enclosure | Yes |
| Shower glass | Yes |
| Closet / wardrobe system | Yes |
| Drywall smooth finish + paint | Yes |
| Bedroom flooring / bath flooring | Yes |
| Trim / doors / hardware | Yes |
| Cleanup + punch list | Yes |
| Exclusions / alternates | Required |

# 14. Materials Cost Estimates

| Item | Allowance |
| --- | --- |
| Bath floor tile | $____ / sf |
| Shower wall tile or slab-look panel | $____ / sf |
| Shower floor tile | $____ / sf |
| Waterproofing system | $____ |
| Shower drain | $____ |
| Shower glass | $____ |
| Freestanding tub | $____ |
| Tub filler | $____ |
| Shower fixture package | $____ |
| Toilet | $____ |
| Vanity cabinet | $____ |
| Vanity countertop | $____ |
| Sinks/faucets | $____ |
| Mirrors / medicine cabinets | $____ |
| Lighting fixtures | $____ |
| Closet / wardrobe system | $____ |
| Bedroom flooring | $____ / sf |
| Paint / smooth finish | $____ |

# 15. Alternates to Price Separately

Option A - Balanced Bath / Closet Plan if lower cost than Option B.

Option C - Closet-Priority Plan if materially cheaper.

Hallway reclaim / bedroom wall reconfiguration to create larger bedroom footprint.

Custom wardrobe wall upgrade versus standard closet system.

Wet-room shower/tub concept versus separate tub + shower.

Curbless shower with linear drain.

Heated bath floor.

Premium slab shower/fireplace-style bath wall versus large-format tile.

Freestanding tub if not included in base.

Exterior primary bath addition - future alternate only, not base scope.

# 16. Required Hold Points

| Hold Point | Approval Requirement |
| --- | --- |
| Layout walk | Before rough work: tape/mark bedroom wall moves, bath layout, vanity, tub, shower, WC, closet, lighting, doors. Owner/designer approval required. |
| Post-demo review | Review exposed framing, slab, plumbing, electrical, HVAC, and any structural constraints before rebuilding. |
| MEP rough-in review | Approve plumbing, valves, drains, outlets, switches, lighting, exhaust, and HVAC before closing walls. |
| Waterproofing/flood test | Approve shower waterproofing and flood test before tile. |
| Tile layout approval | Approve dry layout or layout drawing before visible tile install. |
| Cabinet/vanity approval | Approve vanity/closet millwork before countertop template. |
| Final punch walk | Final payment only after punch list, cleaning, fixture operation, and visible defects are resolved. |

# 17. Interior Design Team Deliverables

Final primary suite layout comparing Option B, Option A, and any hallway-reclaim configuration.

Furniture plan for bedroom showing king bed, nightstands, circulation, and wardrobe/closet function.

Bath plan with tub, shower, vanity, WC, closet, door swings, and clearances.

Bath elevations: vanity wall, shower/tub wall, WC/closet condition.

Tile layout plan including niche, bench, tub wall, shower floor, and visible cut locations.

Lighting plan: vanity, shower, tub, general, accent, closet, switches/dimmers.

Material palette: flooring, tile, counters, cabinetry, metal finishes, paint/plaster, glass.

Closet/wardrobe design: storage zones, drawer/hanging/shelf layout, lighting, finish.