Main Living / Public Zone Spec

North Scottsdale Desert-Modern Renovation

Contractor + Interior Design Bid Alignment Document

June 2026

Core Vision Create a warm North Scottsdale desert-modern great room where the buyer walks in and immediately understands the plan: dining to the right, TV/media to the left, center living ahead, and a powerful fireplace/pool/backyard focal wall at the rear.

Prepared for contractor bidding, interior design alignment, and scope control. All dimensions and structural conditions must be field verified before construction.

# At-a-Glance Direction

This document defines the intended design, base scope, bid alternates, material standards, and workmanship expectations for the main public living area only.

| Zone | Function | Design Direction | Must Not Happen |
| --- | --- | --- | --- |
| Right of entry | Dining / hospitality | Oval/racetrack table, 6-8 chairs, sideboard/dry bar, statement chandelier | No TV, random hutch, farmhouse chandelier, or blocked circulation |
| Left of entry | TV / media lounge | Low-profile sofa/sectional, clean media wall, concealed wiring | No exposed cords, oversized recliner layout, or TV dominating entry sightline |
| Center | Living / conversation | Furniture oriented toward fireplace or rear glass, warm materials, open circulation | No furniture barricade, no random formal sitting room |
| Rear wall | Resort focal point | Option A: redesigned fireplace + enlarged flanking doors/windows. Option B: fireplace removed + full glass wall | No undersized slider, cheap white vinyl, or weak pool connection |
| Entire zone | Cohesive public space | Continuous flooring, smooth walls, dark beams, warm lighting | No gray flip finishes, mismatched texture, patchwork flooring, or heavy Tuscan detailing |

Base Recommendation Price Option A as the base construction plan: keep and redesign the fireplace, then enlarge the flanking rear doors/windows as much as structure and budget reasonably allow. Price Option B - removing the fireplace for a full rear glass wall - as a premium alternate.

# 1. Executive Summary

The public living area should be redesigned as a single connected resort-style great room, not a set of chopped-up formal rooms. The buyer should walk in and immediately understand three zones: dining to the right, TV/media lounge to the left, and center living ahead in front of the fireplace or rear glass focal wall.

The design must sell the North Scottsdale lifestyle: warm desert modernism, indoor-outdoor living, pool visibility, hospitality-grade entertaining, natural materials, calm lighting, and a high-end but restrained finish package.

The contractor is not being hired simply to make the room nicer. The contractor is being hired to execute a specific result: a warm desert-modern great room with clear zoning, continuous finishes, a premium rear-resort connection, and workmanship clean enough to support a luxury resale product.

## One-Sentence Vision

Vision Statement Create a warm Sonoran Resort Modern great room with right-side dining, left-side TV/media, center living, and a major rear pool-facing fireplace/glass moment that makes the backyard feel like the emotional payoff of the house.

## Design Style: Sonoran Resort Modern

The correct style lane is organic desert contemporary: smooth plaster-like walls, warm limestone tones, dark restored beams, warm woods, blackened bronze accents, matte/honed stone, layered lighting, and low visual clutter.

| Use | Avoid |
| --- | --- |
| Smooth warm plaster-like walls | Orange-peel texture called smooth |
| Warm limestone / sand / bone / taupe palette | Cold gray, stark white, or yellow beige |
| Continuous large-format public-area flooring | Patchwork flooring or gray LVP |
| Dark restored beams | Glossy orange stain or whitewashed beams unless approved |
| Bronze / blackened metal / warm metal accents | Random mixed metals or dated oil-rubbed Tuscan hardware |
| Honed stone, limestone-look porcelain, subtle quartzite | Shiny generic marble, busy granite, stacked stone |

# 2. Proposed Zone Plan

Use the following organizing logic for design, construction, furniture planning, electrical rough-in, and final staging.

| Position from Front Door | Zone | Primary Use | Key Construction Implications |
| --- | --- | --- | --- |
| Right | Dining / hospitality zone | Dining table, sideboard/dry bar, statement chandelier | Center lighting to table; keep circulation clear; provide clean wall for sideboard/art |
| Left | TV / media lounge | Comfortable television area | Conceal wiring; approve TV wall before rough-in; avoid entry-view domination |
| Center | Living / conversation zone | Main seating, fireplace/pool orientation | Maintain path to rear; coordinate rug/furniture plan with lighting and outlets |
| Rear | Resort focal wall | Fireplace + glass, or full glass wall | Engineering, doors/windows, waterproofing, stucco patching, flooring thresholds |
| Through entire area | Circulation spine | Front door to rear pool connection | Maintain 42 in minimum primary path; 48 in preferred |

## Primary Circulation Rule

Maintain a clear visual and physical path from the front door through the center of the public space toward the rear pool connection. Do not allow furniture, posts, media cabinets, dining chairs, or sideboards to pinch or confuse this route.

Primary circulation: 42 inches minimum, 48 inches preferred.

Secondary circulation: 36 inches minimum.

Door swings, chair pullback, media furniture, and sofa backs must be checked against these clearances.

# 3. Rear Wall / Fireplace Decision

The rear wall is the largest design fork and the largest perceived-value opportunity in the public zone. Contractors must price Option A as the base bid and Option B as a separate alternate.

## Option A - Base Bid: Keep Fireplace + Enlarge Flanking Rear Doors/Windows

The fireplace remains the central architectural anchor, but the two sides of the fireplace are opened up more aggressively to the backyard/pool. The finished wall should read as a plaster/stone fireplace centerpiece with large glass openings to the pool on both sides.

Demo existing dated fireplace finish as needed.

Inspect fireplace/firebox/gas/venting condition before finish work.

Rebuild fireplace wall in smooth plaster, limestone, limestone-look slab/panel, or approved warm stone.

Remove dated mantel profiles and heavy decorative elements.

Install or enlarge rear door/window openings on both sides of fireplace, subject to engineering and field conditions.

Align head heights of rear doors/windows where feasible.

Use matching black, bronze, or dark metal frames.

Provide proper flashing, waterproofing, sill/threshold detailing, exterior stucco patching, and interior finish integration.

Conceal TV/electrical wiring in approved TV/media zone, not randomly over the fireplace unless approved.

### Option A Acceptance Standard

The finished rear wall must feel intentional, warm, and resort-oriented. The fireplace should anchor the room without blocking the pool lifestyle. The new doors/windows must materially improve the indoor-outdoor connection, not merely replace old openings with slightly newer versions.

## Option B - Alternate Bid: Remove Fireplace + Full Rear Glass / Door / Window Wall

This is the more dramatic option. It removes the fireplace and converts the rear wall into a resort-style glass composition facing the backyard/pool. This option is only worth doing if it produces a genuinely superior pool-view transformation with high-quality glass and clean structural execution.

Full fireplace removal, including gas line termination/capping by licensed trade.

Firebox/chimney/chase removal or modification, including roof patching if applicable.

Structural engineering for new wide glass opening.

New beam/header/posts/returns and load path as required.

New large glass door/window composition: multi-slide, large slider, French/steel-style doors, or fixed/operable glass system.

Low-E tempered glass as required, with dark bronze/black/warm metal frame.

Flashing, waterproofing, sill pan/threshold, exterior stucco patching, and interior plaster/drywall finish.

Flooring patch/continuity, electrical relocation, HVAC impact review, paint, and trim.

### Option B Decision Rule

Select Option B only if the backyard/pool view becomes the dominant asset of the room and the structural/roof/exterior implications price cleanly. Do not remove the fireplace merely to install an average slider. A mediocre glass wall is worse than a strong fireplace with expanded flanking doors.

# 4. Zone-by-Zone Design Spec

## Zone 1 - Right-Side Dining Area

Purpose: formal entertaining and hospitality zone visible from entry. This should tell the buyer the house hosts dinners, wine nights, family gatherings, and resort-style entertaining.

| Element | Specification |
| --- | --- |
| Table | Oval or racetrack dining table, 84-96 in long, 40-46 in wide; 6 chairs standard, 8 only if clearances work |
| Clearance | 36 in minimum from table edge to obstruction; 42-48 in preferred on primary paths |
| Lighting | Statement chandelier centered to table, not old box if old box is wrong; dimmer required; 2700K-3000K |
| Furniture | Warm wood table; slim upholstered chairs in ivory, sand, camel, oatmeal, or taupe |
| Wall | Low sideboard/dry bar with large art or textured piece above |
| Avoid | TV, farmhouse chandelier, crystal chandelier, heavy Tuscan iron, random hutch, oversized table blocking circulation |

## Zone 2 - Left-Side TV / Media Lounge

Purpose: the actual TV-watching area. It should be comfortable and useful without hijacking the whole room.

| Element | Specification |
| --- | --- |
| TV | 65-75 in TV, Frame-style or clean recessed approach preferred; centerline generally 42-48 in AFF unless seating plan dictates otherwise |
| Media wall | Low console or minimal built-in; warm wood/plaster; concealed wiring; no exposed cords |
| Seating | Low-profile sofa or sectional; one accent chair/ottoman if space allows |
| Lighting | No glare on TV; low-level lamps/sconces optional; clean switch/outlet layout |
| Avoid | Bulky recliner layout, black leather bachelor-pad look, wall of cheap built-ins, TV dominating the entry view |

## Zone 3 - Center Living / Conversation Area

Purpose: the emotional center of the public room. It should feel like the place to sit, see the fire, see the pool, have drinks, and relax.

| Element | Specification |
| --- | --- |
| Furniture | Sofa + two lounge chairs, two facing sofas, or sectional + chairs depending on final measurements |
| Orientation | Toward fireplace if Option A; toward backyard/glass wall if Option B |
| Rug | 9x12 minimum; 10x14 preferred if space supports it |
| Coffee table | 42-54 in organic, round, or soft rectangle; wood or stone |
| Clearance | 42-48 in main circulation; 36 in minimum secondary routes |

## Zone 4 - Rear Resort Connection

Purpose: the ARV driver. The buyer should feel the backyard and pool from inside the house.

Use black, bronze, or dark-frame glass doors/windows.

Prioritize taller/wider openings where structurally and financially reasonable.

Use low-E glass and tempered glass where required.

Blend interior returns and exterior stucco patching cleanly.

Avoid cheap white vinyl, mismatched frames, undersized glass, rough threshold details, or water-intrusion risk.

# 5. Material and Finish Schedule

| Category | Base Specification | Prohibited / Avoid |
| --- | --- | --- |
| Flooring | Large-format warm limestone-look porcelain; 24x48 preferred; matte/honed; tight color-matched grout; continuous through public spaces | Gray LVP, glossy tile, orange/red tile, busy travertine, tile over unflattened slab |
| Walls | Smooth warm plaster-like finish; warm off-white, bone, limestone, soft sand | Orange peel, mismatched texture, cold white, gray, visible patching |
| Ceiling | Warm off-white smooth finish; patching must disappear under normal lighting | Rough patches, texture mismatch, off-center fixture locations |
| Beams | Preserve and refinish existing beams in matte dark walnut, espresso brown, or blackened brown | Gloss orange stain, bright white paint unless approved, unfinished construction damage |
| Baseboards | Flat stock 5.5-6.5 in preferred; warm white or wall color | Skinny builder base, ornate colonial/Tuscan trim |
| Fireplace | Smooth plaster, honed limestone, limestone-look slab/panel, subtle warm quartzite | Shiny generic marble, busy veining, stacked stone, ornate mantel |
| Rear glass | Black/bronze/dark metal frames, low-E glass, modern hardware, clean threshold | White vinyl, mismatched frames, undersized glass, flimsy doors |
| Metals | Dark bronze, blackened metal, aged brass sparingly | Random mixed finishes, cheap bright chrome everywhere |

## Flooring Installation Standard

Large-format tile must not be installed over a wavy or unprepared slab. Contractor must include substrate prep, flattening, crack treatment where needed, layout approval, and clean grout execution. No spot-bonding. No visible lippage in normal daylight. No transition strips used to hide poor planning.

## Wall / Ceiling Finish Standard

Smooth means smooth. Public-facing walls, the fireplace wall, rear door/window returns, and any structural opening/header areas must not show patch lines, old wall ghosts, orange peel, visible seams, or wavy drywall under normal daylight and installed lighting.

# 6. Lighting Spec

Lighting must be treated as a design system, not random fixture replacement. All public-room lighting should be warm, layered, and dimmable.

| Area | Specification |
| --- | --- |
| Color temperature | 2700K-3000K; 90+ CRI preferred; dimmers required in public/living/dining areas |
| Dining | Statement chandelier centered on dining table; warm bronze/dark metal/linen/woven/plaster fixture; not old-box driven if old box is wrong |
| TV/media | No glare on TV; concealed wiring; optional lamps/sconces; clean switch/outlet locations |
| Center living | Recessed/gimbal lighting only as needed; lamps/outlets planned; no harsh overhead-only lighting |
| Rear glass / pool side | Avoid nighttime glass glare; exterior patio lighting should read warm and resort-like |
| Avoid | Cold blue LEDs, too many cans, random fixture styles, off-center chandelier, exposed cords |

# 7. Contractor Base Scope of Work

Base bid shall include the following work for the main public living zone.

Field verification of room dimensions, ceiling heights, beam locations, door/window openings, fireplace conditions, electrical, and HVAC.

Protection, dust control, demolition, and haul-off for affected public-zone work.

Structural evaluation for wall, fireplace, rear opening, or ceiling modification where applicable.

Engineering and permit coordination for structural work, if required.

Public-zone reconfiguration into right dining, left TV/media, center living, and rear resort connection.

Option A fireplace remodel as the base rear-wall plan.

Option A rear pool-facing door/window enlargement or replacement.

Option B fireplace removal and full rear glass wall priced separately as an alternate.

Continuous large-format flooring through the public zone.

Smooth wall/ceiling finish in affected public areas.

Beam repair/refinishing.

Electrical and lighting modifications, including concealed TV/media wiring in approved left TV zone.

HVAC/register adjustments as required by the work.

Painting, trim/baseboards, exterior stucco patching related to rear openings, final cleanup, and punch list.

## Required Bid Alternates

Alternate 1 - Full rear glass wall: fireplace removal, gas/vent termination, roof/chase patch if applicable, structural header/beam, engineering, permits, door/window system, flashing, stucco, finishes, and electrical/HVAC adjustments.

Alternate 2 - Premium rear door system: upgrade standard doors/sliders to premium multi-slide or steel-style system.

Alternate 3 - Full Level-5 public wall finish: upgrade all public-room walls/ceilings to Level 5 or equivalent smooth finish.

Alternate 4 - Natural stone public flooring: upgrade limestone-look porcelain to real limestone/travertine/shellstone.

Alternate 5 - Premium fireplace slab: upgrade fireplace from plaster/large-format porcelain to slab limestone/quartzite.

Alternate 6 - Custom media wall: minimal low built-in for left TV zone only; warm wood/plaster; must not dominate entry sightline.

## Required Allowances

Contractor must list allowances. 'All materials included' is not acceptable unless materials are named or allowance amounts are provided.

| Item | Allowance |
| --- | --- |
| Public-zone floor tile | $____ / sf |
| Floor prep / leveling | $____ |
| Rear doors/windows Option A | $____ |
| Full glass wall Option B | $____ |
| Fireplace finish material | $____ |
| Fireplace labor/framing | $____ |
| Smooth wall/ceiling finish | $____ |
| Paint | $____ |
| Dining chandelier | $____ |
| Recessed lighting | $____ |
| Switches/dimmers | $____ |
| TV/media electrical | $____ |
| Baseboards/trim | $____ |
| Beam refinishing | $____ |
| Exterior stucco patching | $____ |

# 8. Quality / Acceptance Standards

Work is not accepted merely because it is installed. It must be clean, aligned, coordinated with the approved layout, and consistent with the design intent.

| Category | Acceptable | Not Acceptable |
| --- | --- | --- |
| Structural / openings | Engineered/permitted where required; beam/header installed per plans; opening plumb, square, smooth, and intentional | Unpermitted structural removal; hidden uninspected beam; crooked opening; casing used to hide bad drywall; leftover arch ghosting |
| Flooring | Substrate flattened; layout approved; clean grout joints; minimal lippage; continuous through public zone; no hollow tiles | Tile over wavy slab; spot bonding; sliver cuts in focal sightlines; grout haze; crooked layout |
| Walls / ceiling | Smooth consistent finish; no patch lines, visible seams, or texture mismatch under normal lighting | Orange peel; drywall seams; rough ceiling patches; fastener pops; patchwork around openings |
| Doors / windows | Level/plumb/square; flashed/waterproofed; smooth operation; aligned head heights where possible; clean stucco repair | Cheap substituted product; rough stucco patch; sticky doors; water intrusion risk; mismatched frame finishes |
| Fireplace | Correct proportions; warm matte material; no visible wires; clean hearth/bench detail; aligned with living zone | Shiny marble by default; stacked stone; ornate Tuscan detail; TV dominating fireplace; awkward bench |
| Electrical / lighting | Approved fixture locations; concealed TV wiring; dimmers; warm 2700K-3000K lighting; aligned devices | Exposed cords; off-center fixtures; cold lighting; random switch placement; TV wiring in wrong zone |

# 9. Interior Design Team Deliverables

The interior design team should use this spec to produce selections and drawings that turn the construction direction into a finished room plan.

Final furniture plan for right dining / left TV / center living.

Lighting plan and decorative fixture selections.

Public-zone finish palette.

Rear wall recommendation: Option A vs Option B after site measurements and contractor pricing.

Fireplace elevation for Option A.

Glass wall elevation for Option B.

Flooring selection and grout color.

Wall color / plaster finish.

Beam finish sample.

Furniture/staging direction for listing photos.

## Listing Photo Priorities

Front door view through great room to rear pool connection.

Right dining zone with chandelier and sideboard.

Left TV/media lounge as a useful secondary space.

Center living zone with fireplace or glass-wall focal point.

Rear pool connection during warm daylight and evening lighting conditions.